- PROBLEM STATEMENT AND REQUIREMENTS

1.1 Problem statement

The manual operation of streetlight systems often leads to significant energy wastage when lights are left on during daylight hours, or safety hazards when they aren't turned on promptly at night.

Existing automated systems frequently suffer from:

- Fixed Thresholds: They cannot adapt to varying weather conditions and seasonal variations in daylight hours (like heavy overcast or storms).

- Lack of Flexibility: There is often no way to bypass the sensor for maintenance or emergency situations.

- Sensitivity Issues: Rapid flickering caused by momentary shadows or car headlights.

This project aims to design an automatic, reliable, adjustable controller that optimizes energy usage by responding to ambient light levels while providing manual control for maintenance.

1.2 Operating Environment

The controller will be installed in weatherproof enclosures mounted on streetlight poles or utility boxes. It must withstand temperature variations, humidity, dust, and occasional mechanical vibrations. The system requires protection against outdoor environmental conditions including rain, UV exposure, and electromagnetic interference from nearby power lines.

1.3 System Inputs & Outputs

1.3.1 Inputs

- Light Sensor: A Light Dependent Resistor (LDR) or a Photodiode to monitor ambient lux levels.

- Threshold Adjustment: A potentiometer (variable resistor) to allow the user to set the specific "darkness" level for activation.

- Manual Override: A 3-position physical toggle switch or push-button (Auto/Manual-ON/Manual-OFF).

1.3.2 Outputs

- Actuator: An electromagnetic Relay or Solid-State Relay (SSR) to switch the high-voltage AC lamp circuit.

- Status Indicators: LEDs to show system status (e.g., Green for "System Active," Red for "Lamp On").

1.3.3 Power Supply Assumptions

- Logic Level: 5V or 12V DC (via a step-down transformer and regulator).

- Load Level: 110V/220V AC mains for the actual street lamps.

1.4 Performance Requirements

| Parameter | Requirement |
| --- | --- |
| Light Detection Range | 1 lux – 1000 lux |
| Threshold Adjustment | Threshold should be adjustable between 10 lux (deep dusk) and 100 lux (overcast day). |
| Response Time | ≤ 5 seconds switching delay after threshold detection with a 5–10 second delay (hysteresis) to prevent false triggering caused by lightning, car headlights or clouds. |
| Accuracy | ±10% light level measurement |
| Reliability | ≥ 50,000 hours continuous operation (approximately 5.7 years) ≥ 100,000 switching cycles Protection features; - Reverse polarity protection - Overvoltage protection - Waterproof enclosure |
| Power consumption | < 5W (control circuit only, excluding light load) |

- System Block Diagram